BLV Series Motor Driver — Interface Reference

Quick guide for direct CN4 control with Arduino Mega. For BLV620KM, BLV640KM and similar 24 V drivers.

## 1. Connectors

| Connector | Purpose |
| --- | --- |
| CN1 | DC power supply input. Connect 24 V battery (+) and (−). |
| CN2 | 3-phase output to motor. The driver sends PWM voltage here. |
| CN3 | Motor sensor signal. Connect motor sensor cable. |
| CN4 | I/O signal port. D-Sub 15-pin. Used by Arduino to control the driver. |
| CN5 / CN6 | RS-485 communication ports. RJ-45. Used for Modbus control |
| CN7 | Service port for OPX-2A or PC software (MEXE02). Mini-DIN 8-pin. |
| CN8 | Electromagnetic brake wires. Connect to brake-type motor only. |

## 2. CN4 I/O pins

| CN4 Pin | Signal | Function | Logic in code (sink mode) |
| --- | --- | --- | --- |
| 1 | FWD | Forward run command | LOW = motor runs forward |
| 2 | REV | Reverse run command | LOW = motor runs reverse |
| 3 | STOP-MODE | How the motor stops | LOW = decel stop, HIGH = instant stop |
| 4 | M0 | Operation data select | HIGH = uses data No.0 (default) |
| 5 | IN-COM | Common ground for inputs | Connect to Arduino GND |
| 10 | MB-FREE | Manual brake release | HIGH = brake auto operates |
| 12 | VM (analog) | Speed setpoint input | 0 to 5 V DC = 0 to max RPM |

Sink logic note: Arduino pin LOW = signal ON. Arduino pin HIGH = signal OFF. This is set by SW1-No.3 = OFF.

## 3. LED status

| LED | Color | Meaning |
| --- | --- | --- |
| POWER | Green | Lit when CN1 power is OK. |
| ALARM | Red | Blinks when an error is active. Count blinks to read the code. |
| C-DAT | Green | Flashes during RS-485 data exchange (Modbus mode only). |
| C-ERR | Red | Lit/blinking when RS-485 communication error happens. |

## 4. Switch and potentiometer settings (working driver)

| Switch / Pot | Setting | Meaning |
| --- | --- | --- |
| SW1 (1, 2, 3) | All OFF | Sink logic, slow response, 0–5 V analog input range. |
| SW2-3, SW2-5 | ON | RS-485 baud 115,200 bps and Modbus protocol enabled. Not used by CN4 control. |
| SW3 | 1 | Slave address (only used for RS-485). |
| VR1 (Speed) | Free | Internal speed pot. Not used when M0=HIGH and external VM is active. Very important if used the |
| VR2 (Accel) | Fully CCW | Acceleration / deceleration time. Fully CCW = 0.2 sec (fastest). |
| VR3 (Torque) | Fully CW | Torque limit. Fully CW = 200% (maximum). Do not lower. |

## 5. Speed control method

The Arduino sends PWM (0 to 255) on pin 2 (left) or pin 3 (right). The PWM signal becomes a DC voltage from 0 V to 5 V at the VM input on CN4. The motor runs only when FWD or REV is also ON. Speed is set by PWM value: 0 = stop, 255 = max RPM.

## 6. Tested working program

The test code drives two BLV drivers (left and right motor) using the pins shown in section 2. The motors run forward for 3 seconds, then stop for 3 seconds, in a loop. M0 is set HIGH so the driver uses operation data No.0 with the external analog input (VM) as the speed source. MB-FREE is set HIGH so the brake operates automatically (locks when stopped, releases during run).